# Annex: Transformation Parameters by Pathway

| Transformation | Policy Description | BAU | NDC2 Conditional | NDC2 Unconditional (As Planned) | NDC2 Unconditional w/Additional Cost Effective | Candidate NDC3 |
| --- | --- | --- | --- | --- | --- | --- |
| Agriculture – Crops |  |  |  |  |  |  |
| Decrease Exports | Decrease agricultural exports relative to baseline levels by the final time period. | No policy | No policy | No policy | No policy | No policy |
| Expand conservation agriculture | Expand conservation agriculture practices — minimum tillage, permanent soil cover, and species diversification — increasing crop residues retained on the field. | No policy | 50% of crop residues retained on field | No policy | No policy | 50% of crop residues retained on field |
| Improve crop productivity | Apply a fractional increase to crop yield factors per hectare to improve land productivity. | 20% increase in crop yields per hectare | 60% increase in crop yields per hectare | No policy | 60% increase in crop yields per hectare | 70% increase in crop yields per hectare |
| Improve rice management | Reduce methane emissions from rice cultivation through improved water management, fertilization, tillage, and residue management practices. | No policy | No policy | No policy | No policy | 45% reduction in methane emissions from rice cultivation |
| Reduce supply chain losses | Reduce pre-consumer food waste along the agricultural supply chain, lowering production demand for crops. | No policy | No policy | No policy | No policy | 50% reduction in supply chain food losses |
| Agriculture – Soils |  |  |  |  |  |  |
| Improve fertilizer application | Decrease total nitrogen applied through precision fertilizer use without reducing crop yields. | No policy | No policy | No policy | No policy | 20% reduction in fertilizer nitrogen applied |
| Improve lime application | Decrease lime applied to agricultural soils through improved soil management and pH monitoring practices. | No policy | No policy | No policy | No policy | 20% reduction in lime applied to soils |
| Land Use |  |  |  |  |  |  |
| Dec Wetland Loss | Halt wetland loss by increasing the self-transition probability for wetlands to near one, preventing conversion to other land uses. | No policy | No policy | No policy | No policy | No policy |
| Expand silvopasture | Convert a share of pasture to silvopasture (integrated tree–livestock systems), with livestock carrying capacity adjusted to compensate. | No policy | 20% of pasture converted to silvopasture | No policy | No policy | 20% of pasture converted to silvopasture |
| Expand sustainable grazing practices | Decrease soil organic carbon loss in grasslands by expanding sustainable grazing practices across a defined share of pasture area. | No policy | No policy | No policy | No policy | No policy |
| Increase Reforestation | Increase probability of land being converted to secondary forest, raising secondary forest area above baseline by the final time period. | No policy | 10% increase in secondary forest area | No policy | No policy | 30% increase in secondary forest area |
| Plur | Apply a partial land use reallocation factor that redistributes a fraction of projected land use transitions across categories, smoothing extreme shifts in the land use transition matrix. | No policy | 28% land use reallocation factor applied | No policy | No policy | No policy |
| Set Wetlands Minimum | Set a minimum bound on wetlands area using the land use quadratic programming framework, representing protected area targets or restoration commitments for wetland ecosystems. | Minimum wetlands area bound enforced to protect and restore wetland ecosystems | Minimum wetlands area bound enforced to protect and restore wetland ecosystems | Minimum wetlands area bound enforced to protect and restore wetland ecosystems | Minimum wetlands area bound enforced to protect and restore wetland ecosystems | Minimum wetlands area bound enforced to protect and restore wetland ecosystems |
| Stop deforestation | Halt deforestation by setting the primary forest self-transition probability to near one — primary forest is kept as primary forest rather than being converted to other land uses. | No policy | 100% of primary forest remains forested each period (near-zero deforestation rate) | No policy | 100% of primary forest remains forested each period (near-zero deforestation rate) | 100% of primary forest remains forested each period (near-zero deforestation rate) |
| Forestry |  |  |  |  |  |  |
| Increase Sequestration | Increase carbon sequestration in secondary forests to represent gains from sustainable biomass management and forest restoration. | No policy | 10% increase in secondary forest carbon sequestration | No policy | No policy | 15% increase in secondary forest carbon sequestration |
| Livestock |  |  |  |  |  |  |
| Decrease exports | Decrease exports of livestock and livestock products relative to baseline levels by the final time period. | No policy | No policy | No policy | No policy | No policy |
| Increase livestock productivity | Increase livestock carrying capacity (average land productivity) above baseline levels by the final time period. | 21% increase in livestock productivity (animals per unit area) | 30% increase in livestock productivity (animals per unit area) | No policy | No policy | 50% increase in livestock productivity (animals per unit area) |
| Reduce enteric fermentation | Reduce methane emissions from enteric fermentation in ruminant livestock through feed management and/or methagenic vaccines. Default reductions per species: 40% for buffalo, dairy cattle, and non-dairy cattle; 56% for goats and sheep. | No policy | Default rates: 40% reduction (buffalo, cattle), 56% reduction (goats, sheep) | No policy | No policy | Default rates: 40% reduction (buffalo, cattle), 56% reduction (goats, sheep) |
| Livestock – Manure Management |  |  |  |  |  |  |
| Improve manure management for cattle and pigs | Improve manure management for cattle (dairy and non-dairy) and pigs, directing treated manure to: ~59% anaerobic digestion, ~24% daily spreading, and ~12% composting by the final time period. | No policy | No policy | No policy | No policy | Manure treated (cattle, pigs): ~59% anaerobic digestion, ~24% daily spread, ~12% composting |
| Improve manure management for other animals | Improve manure management for other livestock (buffalo, goats, horses, mules, sheep), directing treated manure to: ~48% anaerobic digestion, ~24% composting, ~12% dry lot, and ~12% daily spreading. | No policy | No policy | No policy | No policy | Manure treated (buffalo, goats, horses, sheep): ~48% anaerobic digestion, ~24% composting, ~24% dry lot/daily spread |
| Improve manure management for poultry | Improve manure management for poultry (chickens), directing treated manure to: ~48% anaerobic digestion and ~48% dedicated poultry manure management pathways. | No policy | No policy | No policy | No policy | Poultry manure (chickens): ~48% anaerobic digestion, ~48% dedicated poultry treatment |
| Increase biogas capture at anaerobic decomposition facilities | Increase the fraction of biogas captured at anaerobic livestock manure management facilities. | No policy | No policy | No policy | No policy | 94% of biogas captured at manure facilities |
| Energy – Electricity |  |  |  |  |  |  |
| 95% of electricity is generated by renewables in 2050 | Set a minimum share of electricity generation from renewable sources (geothermal, hydropower, ocean, solar, wind, and tidal) by 2050. | No policy | 95% renewable electricity share by 2050 | No policy | No policy | 95% renewable electricity share by 2050 |
| Clean hydrogen | Set a target share of hydrogen production sourced from green hydrogen (electrolysis) by the final time period. | No policy | No policy | No policy | No policy | 95% of hydrogen from green electrolysis |
| Reduce transmission losses | Upgrade electrical transmission infrastructure to reduce grid transmission losses. The magnitude sets a final-period loss ceiling (final_value_ceiling): only regions whose baseline loss exceeds the target are affected; losses already below the ceiling remain unchanged. A minimum feasible loss floor (min_loss) applies regardless of target. | No policy | Grid losses capped at 6% — applied only where current losses exceed this level | No policy | No policy | Grid losses capped at 6% — applied only where current losses exceed this level |
| Fugitive Emissions |  |  |  |  |  |  |
| Maximize flaring | Replace methane venting at orphan wells and mining facilities with flaring, converting CH₄ to the less potent CO₂. | No policy | No policy | No policy | No policy | 80% of vented methane converted via flaring |
| Minimize leaks | Reduce fugitive methane emission factors across the production, distribution, and transmission of coal, natural gas, and oil through leak detection and repair (LDAR) programs. | No policy | No policy | No policy | No policy | 80% reduction in methane leakage rates (oil, gas, coal sectors) |
| Industry – Energy |  |  |  |  |  |  |
| Fuel switch high- and low-temp thermal processes. | Switch industrial low-temperature heat processes to electricity and high-temperature heat processes to electricity and hydrogen across cement, metals, chemicals, glass, paper, and lime sectors. | No policy | 90% of industrial heat processes switched to low-carbon fuels | No policy | No policy | 90% of industrial heat processes switched to low-carbon fuels |
| Maximize industrial energy efficiency | Increase the average energy efficiency of industrial production processes by fuel type to reduce overall energy demands. | No policy | 10% improvement in industrial energy efficiency | No policy | No policy | 40% improvement in industrial energy efficiency |
| Maximize industrial production efficiency | Reduce industrial end-use energy demand through modernization of production processes (kJ per tonne improvement). | No policy | No policy | No policy | No policy | No policy |
| Industry – Processes |  |  |  |  |  |  |
| Demand management | Reduce overall industrial production demand relative to the baseline by the final time period. | No policy | No policy | No policy | No policy | No policy |
| Reduce Nitrous Oxide emissions | Reduce IPPU emission factors for nitrous oxide (N₂O) generated as a byproduct of industrial processes. | No policy | No policy | No policy | No policy | 10% reduction in industrial N₂O emissions |
| Reduce cement clinker | Substitute clinker in cement production with supplementary cementitious materials (SCMs), LC3 cement, or equivalent processes. | No policy | Clinker fraction capped at 50% of cement production | No policy | No policy | Clinker fraction capped at 50% of cement production |
| Reduce other fluorinated compounds | Reduce emission factors for other fluorinated compounds (SF₆, NF₃, HCFCs) generated as byproducts of industrial processes. | No policy | No policy | No policy | No policy | 10% reduction in SF₆, NF₃, and other fluorinated compound emissions |
| Reduce use of HFCs | Reduce IPPU emission factors for hydrofluorocarbons (HFCs) through improved refrigerant management, recovery, and phase-down. | No policy | 10% reduction in HFC emissions from industrial processes | No policy | No policy | 10% reduction in HFC emissions from industrial processes |
| Reduce use of PFCs | Reduce IPPU emission factors for perfluorinated carbons (PFCs) generated as byproducts of industrial processes. | No policy | No policy | No policy | No policy | 10% reduction in PFC emissions from industrial processes |
| Cross-Cutting |  |  |  |  |  |  |
| Change diets | Reduce average per-capita demand for cattle products through dietary shifts toward healthier, lower-emissions diets. | No policy | No policy | No policy | No policy | 30% reduction in per-capita cattle consumption |
| Industrial carbon capture and sequestration | Deploy carbon capture and sequestration (CCS) across applicable industrial facilities excluding energy generation, with a default capture efficacy of 90%. | No policy | No policy | No policy | No policy | Industrial CCS deployed with 90% capture efficacy per facility |
| Buildings |  |  |  |  |  |  |
| Inc Efficiency Heat | Increase the average efficiency of heat energy from biomass through adoption of clean cookstoves and improved heating systems. | No policy | 33% improvement in biomass heating efficiency (clean cookstoves) | 33% improvement in biomass heating efficiency (clean cookstoves) | 33% improvement in biomass heating efficiency (clean cookstoves) | 33% improvement in biomass heating efficiency (clean cookstoves) |
| Increase appliance efficiency | Reduce electricity demand in buildings by increasing the efficiency of electrified appliances and building energy systems. | No policy | No policy | No policy | No policy | 30% reduction in building electricity demand |
| Reduce end-use demand for heat energy by improving building shell | Reduce end-use heat energy demand in buildings through retrofitting, insulation improvements, and smart thermostats. | No policy | 5% reduction in building heat energy demand | 5% reduction in building heat energy demand | 5% reduction in building heat energy demand | 40% reduction in building heat energy demand |
| Shift Fuel Heat Strategy Ndc 25 Elec Cm |  | No policy | No policy | No policy | No policy | No policy |
| Shift Fuel Heat Strategy Ndc 25 Elec Res |  | No policy | No policy | No policy | No policy | No policy |
| Switch to electricity for heat using heat pumps, electric stoves, etc. | Electrify building heat demand (space heating, water heating, and cooking) using heat pumps, electric stoves, and equivalent equipment. | No policy | No policy | No policy | No policy | 90% of building heat demand electrified |
| Transport – Demand |  |  |  |  |  |  |
| Reduce demand for transport | Reduce aggregate public and private transportation demand through urban planning, congestion pricing, and demand management policies. | No policy | No policy | No policy | No policy | 50% reduction in total transport demand |
| Transport – Supply |  |  |  |  |  |  |
| Electrify light duty road transport | Set a minimum share of the light-duty vehicle fleet (private cars and light trucks) fueled by electricity | No policy | 70% of light-duty vehicle fleet electrified | No policy | No policy | 70% of light-duty vehicle fleet electrified |
| Electrify rail | Electrify rail transportation (freight and passenger) by setting a target share of rail demand fueled by electricity. | No policy | No policy | No policy | No policy | 25% of rail transport electrified |
| Fuel switch maritime | Shift maritime transportation demand away from fossil fuels toward hydrogen and electricity. | No policy | No policy | No policy | No policy | 140% of maritime transport shifted to low-carbon fuels |
| Fuel switch medium duty road transport | Shift medium-duty vehicles (heavy freight, regional, and public transport) from fossil fuels to electricity and hydrogen. | No policy | 70% of medium-duty fleet shifted to electricity or hydrogen | No policy | No policy | 140% of medium-duty fleet shifted to electricity or hydrogen |
| Increase electric transportation energy efficiency | Improve the on-road efficiency of electric vehicles through regulation and technology standards. | No policy | No policy | No policy | No policy | 50% improvement in electric vehicle fuel efficiency |
| Increase non-electric transportation energy efficiency | Improve the on-road efficiency of non-electric vehicles (fossil-fuel ICEs and other fuel types) through fuel standards and regulation. | No policy | 80% improvement in non-electric vehicle fuel efficiency | No policy | No policy | 100% improvement in non-electric vehicle fuel efficiency |
| Increase occupancy for private vehicles | Increase the average occupancy rate of private light-duty vehicles through carpool lanes and incentive programs. | No policy | No policy | No policy | No policy | 25% increase in average private vehicle occupancy |
| Mode shift freight | Shift a share of aviation and road freight demand to freight rail by the final time period. | No policy | 20% of road and air freight shifted to rail | No policy | No policy | 40% of road and air freight shifted to rail |
| Mode shift passenger vehicles to others | Shift a share of passenger road demand to human-powered transport, powered bikes, and public transit. | No policy | 30% of private road trips shifted to public or active transport | No policy | No policy | 60% of private road trips shifted to public or active transport |
| Mode shift regional passenger travel | Shift shares of regional aviation and light-duty road demand to heavy-duty road transport. | No policy | No policy | No policy | No policy | 10% of regional aviation, 20% of light-duty road demand shifted to heavy road |
| Transport – Wastewater |  |  |  |  |  |  |
| Increase biogas capture | Increase the fraction of biogas captured at anaerobic wastewater treatment facilities (advanced and secondary anaerobic). | No policy | No policy | No policy | No policy | 85% of biogas captured at wastewater treatment facilities |
| Increase septic compliance | Increase compliance with pumping schedule and maintenance requirements for rural septic tanks to preserve full emissions and health benefits. | No policy | 90% of septic tanks meeting maintenance compliance standards | No policy | No policy | 90% of septic tanks meeting maintenance compliance standards |
| Wastewater |  |  |  |  |  |  |
| Improved industrial wastewater treatment | Restructure industrial wastewater treatment to default targets: 80% in advanced anaerobic facilities, 10% secondary aerobic, and 10% secondary anaerobic. Targets may be overridden via dict_magnitude. | Target mix: 80% advanced anaerobic, 10% secondary aerobic, 10% secondary anaerobic | Target mix: 80% advanced anaerobic, 10% secondary aerobic, 10% secondary anaerobic | Target mix: 80% advanced anaerobic, 10% secondary aerobic, 10% secondary anaerobic | Target mix: 80% advanced anaerobic, 10% secondary aerobic, 10% secondary anaerobic | Target mix: 80% advanced anaerobic, 10% secondary aerobic, 10% secondary anaerobic |
| Improved rural wastewater treatment | Restructure rural wastewater treatment so that 100% is treated in septic tanks (default target). Target may be overridden via dict_magnitude. | Target mix: 100% treated in septic tanks | Target mix: 100% treated in septic tanks | Target mix: 100% treated in septic tanks | Target mix: 100% treated in septic tanks | Target mix: 100% treated in septic tanks |
| Improved urban wastewater treatment | Restructure urban wastewater treatment to default targets: 30% advanced anaerobic, 30% advanced aerobic, 20% secondary anaerobic, and 20% secondary aerobic. Targets may be overridden via dict_magnitude. | Target mix: 30% advanced anaerobic, 30% advanced aerobic, 20% secondary anaerobic, 20% secondary aerobic | Target mix: 30% advanced anaerobic, 30% advanced aerobic, 20% secondary anaerobic, 20% secondary aerobic | Target mix: 30% advanced anaerobic, 30% advanced aerobic, 20% secondary anaerobic, 20% secondary aerobic | Target mix: 30% advanced anaerobic, 30% advanced aerobic, 20% secondary anaerobic, 20% secondary aerobic | Target mix: 30% advanced anaerobic, 30% advanced aerobic, 20% secondary anaerobic, 20% secondary aerobic |
| Waste – Solid |  |  |  |  |  |  |
| Biogas for energy production | Increase the fraction of captured biogas converted to energy use. | No policy | No policy | No policy | No policy | 95% of captured biogas converted to energy |
| Consumer food waste reduction | Reduce per-capita food waste generation at the consumer level. | No policy | 30% reduction in per-capita consumer food waste | No policy | No policy | 45% reduction in per-capita consumer food waste |
| Incineration for energy production | Increase the fraction of incinerated solid waste used for energy recovery (waste-to-energy). | No policy | No policy | No policy | No policy | 170% of incinerated waste used for energy recovery |
| Increase biogas capture | Increase the fraction of biogas captured from landfills and anaerobic digesters. | No policy | 85% of biogas captured from landfills and digesters | 85% of biogas captured from landfills and digesters | 85% of biogas captured from landfills and digesters | 95% of biogas captured from landfills and digesters |
| Increase composting and biogas | Increase the fraction of organic waste (yard waste, food waste, and sludge) treated in anaerobic digesters and compost facilities. | No policy | 48% of organic waste to anaerobic digestion, 48% to composting | 48% of organic waste to anaerobic digestion, 48% to composting | 48% of organic waste to anaerobic digestion, 48% to composting | 48% of organic waste to anaerobic digestion, 48% to composting |
| Increase landfilling | Increase the fraction of solid waste — not recycled, composted, or digested — directed to managed sanitary landfills (away from open dumping). | No policy | 100% of residual solid waste directed to managed landfills | 100% of residual solid waste directed to managed landfills | 100% of residual solid waste directed to managed landfills | 100% of residual solid waste directed to managed landfills |
| Increase recycling | Increase the fraction of recyclable solid waste that is recycled, reducing landfill decomposition and virgin material production. | No policy | 95% of recyclable waste recycled | No policy | No policy | 95% of recyclable waste recycled |
| Carbon Capture & Sequestration |  |  |  |  |  |  |
| Increase direct air capture | Deploy Direct Air Capture (DAC) technology to remove CO₂e directly from the atmosphere by 2050. | No policy | No policy | No policy | No policy | No policy |