Integrated Energy Resource Master Plan (IERMP)

25 July 2025

Submitted to Ministry of Energy and Mineral Development

Energy efficiency (EE) and demand side management (DSM) measures report

# Contributors

This report was prepared by:

Economic Consulting Associates Limited

41 Lonsdale Road, London NW6 6RA,
United Kingdom

tel:	+44 20 7604 4546

fax:	+44 20 7604 4547

www.eca-uk.com

Norconsult AS

Vestfjordgaten 4, 1338 Sandvika.

Norway

+ 47 67 57 10 00

www.norconsult.com

Research for Transformation and Development

Plot 8-10 Kampala Road, Uganda House, 5th Floor,

P.O. Box 25582, Kampala,

Uganda

+256-776-130913

www.rtdug.org

This report (including any enclosures and attachments) has been prepared for the exclusive use and benefit of the Ministry of Energy and Mineral Development Uganda and solely for the purpose for which it is provided. Unless we provide express prior written consent, no part of this report should be reproduced, distributed, or communicated to any third party. We do not accept any liability if this report is used for an alternative purpose from which it is intended, nor to any third party in respect of this report.

# Contents

Contributors	1

Contents	2

Tables and figures	2

Abbreviations and acronyms	4

1	Introduction	6

2	Review of current EE and DSM measures in Uganda	8

3	EE and DSM policies in Uganda	13

4	EE and DSM prospects in Uganda	22

5	EE and DSM candidate options in Uganda	28

6	Candidate EE and DSM measures to be analysed in the ERMP	36

Annexes	38

A1	Electricity access targets	38

A2	Capital costs for the implementation of measures	38

# Tables and figures

Tables

Table 1  Population and households	8

Table 2  Number of households by Main Source of Energy for Lighting and Cooking	9

Table 3  Power sales by distribution companies to different consumers (GWh)	10

Table 4  DR measures	22

Table 5  Energy demand for lighting per household	24

Table 6  Energy consuming appliances used by households	25

Table 7  Electricity savings, cost, and lifetime of candidate energy efficient appliances to be used in the residential sector (ranked by incremental cost of savings)	28

Table 8  Electricity savings, cost, and lifetime of candidate energy efficient appliances to be used in the commercial sector (ranked by incremental cost of savings)	28

Table 9  Electricity savings, cost, and lifetime of candidate energy efficient appliances to be used in the industrial sector (ranked by incremental cost of savings)	29

Table 10  Technical and economic potential savings by measure in 2030, ranked by end-user benefit-to-cost ratio, in each sector	31

Table 11  Financing sources for strategic partnerships	35

Table 12  Candidate EE and DSM measures for the ERMP	36

Table 13  Policy targets for electricity access	38

Table 14  Capital costs for the implementation of EE and DSM measure	38

Figures

Figure 1  Overall approach for the development of the IERMP in Uganda	6

Figure 2  Total final energy consumption by fuel and sector, 2023	8

Figure 3  Total Final energy consumption by sector	9

Figure 4  Electricity and clean cooking access in Uganda, 2000-2022	11

Figure 5  Electricity and clean cooking access rates in the Energy and Stated Policies Scenario, 2010-2050	13

Figure 6  Share of population with electricity access by technology, 2021-2050	14

Figure 7  Share of households by cooking technology, 2021-2050	14

Figure 8  Building sector final energy consumption by fuel and sub-sector, 2021-2050	15

Figure 9  Agriculture sector final energy consumption by fuel, 2021-2050	15

Figure 10  Industry energy demand by fuel and sub-sector, 2021-2050	16

Figure 11  Electricity demand by sector, 2010-2050	16

Figure 12  Top Measures for Achievable Economic Potential, Medium Case Prioritized by End-User Cost of Conserved Energy	33

# Abbreviations and acronyms

| AfDB | African Development Bank |
| --- | --- |
| CUL | CNOOC Uganda Limited |
| DSM | Demand side management |
| EAPP | Eastern African Power Pool |
| ECA | Economic Consulting Associates |
| EE | Energy Efficiency |
| ERA | Energy Regulatory Authority |
| ESIA | Environmental and Social Impact analysis |
| ETP | Energy Transition Plan |
| EV | Electric vehicles |
| GDP | Grid Development Plans |
| HIS | Householder Income and Expenditure Survey |
| IEA | International Energy Agency |
| IRP | Integrated Resource Plan |
| IERMP | Integrated Energy Resource Master Plan |
| KCCA | Kampala Capital City Authority |
| LFO | Light Fuel Oil |
| LPG | Liquefied Petroleum Gas |
| MEMD | Ministry of Energy and Mineral Development |
| MEPS | Minimum energy performance standards |
| NDC | Nationally Determined Contribution |
| NDP | National Development Plan |
| NEP | National Energy Policy |
| PPA | Power Purchase Agreement |
| PPP | Public Private Partnerships |
| PS | Permanent Secretary |
| REP | Renewable Energy Policy |
| RES | Renewable energy source |
| RTD | Research for Transformation and Development Uganda |
| SG&A | Sales, General, and Administrative |
| SDG7 | Sustainable Development Goal 7 |
| UBOS | Uganda Bureau of Statistics |
| UEDCL | Uganda Electricity Distribution Company Limited |
| UEGCL | Uganda Electricity Generation Company Limited |
| UETCL | Uganda Electricity Transmission Company Limited |
| SG&A | Sales, General, and Administrative |
| UBOS | Uganda Bureau of Statistics |
| UEDCL | Uganda Electricity Distribution Company Limited |

# Introduction

The Energy Efficiency (EE) and Demand Side Management (DSM) measures report has been prepared by Economic Consulting Associates (ECA), Norconsult AS and RTDC for the Development of an Integrated Energy Resource Master Plan (IERMP) for Uganda.

This report aims to analyse EE and DSM measures that have been implemented in Uganda, and candidate measures that could be implemented in the future. The identified candidate measures will be evaluated in the least cost planning phase of the project to assess if they are least cost options to include in the IERMP for Uganda. Existing measures are considered in the demand forecast analysis.

The overall approach to develop the IERMP Uganda is illustrated in Figure 1. This Energy Efficiency and Demand Side Management measures report is part of the preparatory studies needed for the development of the IERMP. Other preparatory studies include the Demand forecast report, and the Resource assessment report, which were developed in parallel to this report. The findings of the Preparatory studies set the foundations for the development of least cost plan in the next phase of the project.

Figure 1  Overall approach for the development of the IERMP in Uganda

Source: ECA

The structure of the Energy Efficiency and Demand Side Management measures report is as follows:

| Section 2 | Reviews the current energy demand conditions in Uganda. |
| --- | --- |
| Section 3 | Discusses existing policies regarding EE and DSM measures. |
| Section 4 | Analyses the savings potential of several EE and DSM measures that could be implemented in Uganda. |
| Section 5 | Analyses the ability and economic value to implement several EE and DSM measures in Uganda. |
| Section 6 | Summarises candidate EE and DSM measures that will be considered in the least cost planning analysis of the IERMP. |

# Review of current EE and DSM measures in Uganda

According to the 2024 Census, the population of Uganda was 45.9 million in 2024, increasing by 11.3 million (about 33%) compared to the 2014 level. The number of households has risen to 10.8 million in 2024, from 7,6 million in 2014, and the household size decreased from an average of about 4.7 persons in 2014 to 4.4 persons in 2024. The following table displays the evolution of population and average household size for Uganda in 2014 and 2024.

Table 1  Population and households

|  | 2014 | 2024 | Average annual grow |
| --- | --- | --- | --- |
| Total population (persons) | 34,634,650 | 45,935,046 | 2.9% |
| Total number of households | 7,306,942 | 10,845,119 | 4.0% |
| Average household size (persons) | 4.7 | 4.4 | -0.7% |

Source: Uganda Bureau of Statistics

Total final energy consumption in 2023 was 16,362.58 ktoe. As illustrated in Figure 2, this comprised about 85% biofuels and waste, 13% oil products, and only 2% electricity. The domestic sector is the main energy consumer, accounting for about half (49%) of the total final energy consumption. Commercial and public services account for about 30%, industry 12%, transport 8%, and agriculture, forestry and fishing about 1% of final energy consumption, respectively.

Figure 2  Total final energy consumption by fuel and sector, 2023

Source: 2023 Statistical Abstract, Ministry of Mining and Mineral Resources

As shown in Figure 3, households mainly consumed biofuels and waste, accounting for about 99% of household final energy consumption, with electricity consumption only accounting for about 1% of total final energy consumption. A similar situation appears in commercial and public services. Industry is consuming biofuels and waste (65%), oil products (22%) and electricity (13%), and the transport sector is only consuming oil products.

Figure 3  Total Final energy consumption by sector

Source: 2023 Statistical Abstract, Ministry of Mining and Mineral Resources

According to the 2024 Census, about 25% of households were connected to either the main electricity network or to mini grids (about 2.7 million households). The high use of biofuels and comparatively low use of electricity in households is shown in detail in Table 2. The table also illustrates that the vast majority of households with access to electricity are still using wood and charcoal burning for cooking in both urban and rural households.

Table 2  Number of households by Main Source of Energy for Lighting and Cooking

| Background characteristic | Urban |  | Rural |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- |
| Access to electricity | 2,130,341 | 49.0% | 576,285 | 9.1% | 2,706,626 | 25.3% |
| Main type of cooking fuel |  |  |  |  |  |  |
| Kerosene/paraffin | 35,233 | 0.9% | 48,387 | 0.8% | 83,620 | 0.8% |
| Charcoal | 1,979,431 | 48.6% | 903,557 | 14.7% | 2,882,988 | 28.2% |
| Wood | 1,678,953 | 41.2% | 4,914,957 | 80.0% | 6,593,910 | 64.5% |
| Electricity | 98,488 | 2.4% | 79,241 | 1.3% | 177,729 | 1.7% |
| Biogas | 83,390 | 2.0% | 23,749 | 0.4% | 107,139 | 1.0% |
| Liquefied Petroleum Gas (LPG) | 99,214 | 2.4% | 16,719 | 0.3% | 115,933 | 1.1% |
| Other | 101,768 | 2.5% | 160,894 | 2.6% | 262,662 | 2.6% |
| Main source of energy for Lighting |  |  |  |  |  |  |
| Electricity (grid and mini grid) | 2,130,341 | 49.0% | 576,285 | 9.1% | 2,706,626 | 25.3% |
| Solar | 897,271 | 20.6% | 2,109,461 | 33.2% | 3,006,732 | 28.1% |
| Rechargeable flashlight, mobile torch or lantern | 219,514 | 5.1% | 619,265 | 9.7% | 838,779 | 7.8% |
| Dry battery powered flashlight, torch, or lantern | 267,150 | 6.1% | 971,357 | 15.3% | 1,238,507 | 11.6% |
| Biogas lamp | 10,042 | 0.2% | 14,809 | 0.2% | 24,851 | 0.2% |
| Liquefied Petroleum Gas (LPG) lamp | 75,701 | 1.7% | 181,785 | 2.9% | 257,486 | 2.4% |
| Gasoline lamp | 19,451 | 0.4% | 35,926 | 0.6% | 55,377 | 0.5% |
| Kerosene/ paraffin lamp /tadooba | 276,779 | 6.4% | 721,529 | 11.4% | 998,308 | 9.3% |
| Candle | 226,717 | 5.2% | 438,777 | 6.9% | 665,494 | 6.2% |
| Open fire | 104,004 | 2.4% | 448,865 | 7.1% | 552,869 | 5.2% |
| Others | 119,725 | 2.8% | 234,159 | 3.7% | 353,884 | 3.3% |
| Total | 4,346,695 | 100% | 6,352,218 | 100% | 10,698,913 | 100% |

Source: National Population and Housing Census 2024, Uganda Bureau of Statistics

Table 3 presents the sales of electricity to different customer categories connected to the grid. The average annual electricity consumption per capita for 2023 was about 94 kWh. Considering only the number of households with access to electricity, average household size, and the electricity consumption in the domestic sector, the average annual electricity consumption for 2023 is estimated at about 82 kWh/capita, a very low level by international standards (eg 618 kWh per capita average for Africa, 3,780 kWh for South Africa, 221 kWh for Kenya).

Table 3  Power sales by distribution companies to different consumers (GWh)

| Year | Domestic | Commercial | Medium Industries | Large Industries | Extra Large | Street Lights | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2019 | 701.8 | 385.3 | 507.7 | 894.4 | 759.3 | 1.3 | 3,249.8 |
| 2020 | 734.3 | 358.7 | 461.4 | 818.7 | 898.4 | 1.6 | 3,273.0 |
| 2021 | 784.3 | 398.7 | 487.5 | 917.3 | 996.1 | 1.9 | 3,585.8 |
| 2022 | 873.9 | 453.8 | 557.6 | 1,050.7 | 1,009.3 | 2.1 | 3,947.4 |
| 2023 | 971.2 | 506.2 | 607.0 | 1,187.0 | 1,060.1 | 2.6 | 4,333.9 |

Source: 2023 Statistical Abstract, Ministry of Mining and Mineral Resources

However, according to the Energy Efficiency Roadmap for Uganda, the average electricity consumption of new customers is expected to grow significantly over the coming years.

As observed in the Uganda Energy Transition Plan, although access to electricity (and clean cooking) is still low, there is a significant increase of access rates from less than 5% in 2000 to roughly 45% in 2022 (see Figure 4).

Figure 4  Electricity and clean cooking access in Uganda, 2000-2022

Source: Uganda Energy Transition Plan, IEA, 2023.

#### Recent and current EE and DSM measures under implementation

Several EE and DSM measures had been implemented in the past and were completed with success including:

distribution of compact fluorescent lamps to households in 2007, resulting to a reduction of demand by 32 MW;

installation of energy efficient equipment in public institutions during 2007-2012, resulting to savings of 10 GWh;

financial support (coverage of 50% of the capital cost) for power factor correction during 2011-2014, leading to a demand reduction of 8.6 MW;

financial support (coverage of 50% of the capital cost) for the installation of solar water heaters, during 2011-1014, resulting to peak shaving by 3.9 MW; and

energy management training workshops carried out in 2015.

As of 2017, EE and DSM measures that were under implementation (still on-going) included:

Industrial energy audits – energy audits in more than 50 high-energy-consuming industries and commercial facilities had been carried out under various projects, reducing energy consumption by about 10 GWh.

Efficiency standards and labelling – such standards were developed and issued in 2012 for lighting, refrigerators, freezers, motors, and air conditioners, but the relevant regulations for enforcement had not been issued, hence the standards had not been implemented. This is included in the draft Energy Efficiency and Conservation Bill which is now approved by the Government and expected to be introduced to the Parliament for discussion.

Awareness – mainly the annual Energy Week, a platform for large public announcements, such as LED distribution, and for exhibiting and demonstrating technologies.

City leadership – according to the Roadmap, the Kampala Capital City Authority (KCCA) conducted energy audits for selected municipal buildings and introduced pilot Energy Efficiency measures including occupancy sensors, lighting timers for corridors, and sensor-controlled outdoor lights.

As of 2024, no other new measures are under implementation.

# EE and DSM policies in Uganda

The Energy Transition Plan (ETP) is a strategic roadmap encompassing Uganda’s energy policies for the development and modernisation of the country’s energy sector. The ETP was issued in 2023.

The following are listed as the guiding principles of the ETP. Among them, the first principle concerns mainly improvements to Energy Efficiency and demand side management.

Achieve universal energy access by 2030, in line with Sustainable Development Goal 7 (SDG7);

Modernise the energy system to support economic growth;

Improve energy security and affordability;

Mitigate emissions from the energy sector;

Position Uganda as a regional energy hub.

#### Issues relative to the ETP objective of achieving universal energy access by 2030

##### Access to electricity and clean cooking

Under the existing policy and financing environment, the IEA, in its Stated Policies Scenario (STEPS) assess that by 2030 electricity access will reach 60% of the population (of which 18% with solar multi light systems), and clean cooking will rise to 28% (of which half with improved biomass cookstoves). This is illustrated in Figure 5 and Figure 6.

Figure 5  Electricity and clean cooking access rates in the Energy and Stated Policies Scenario, 2010-2050

Source: Uganda Energy Transition Plan.

Note: Electricity access targets among various policy documents are shown in Annex A1.

Figure 6  Share of population with electricity access by technology, 2021-2050

Source: Uganda Energy Transition Plan

According to the analysis done for the purposes of the Uganda ETP, per capita electricity consumption almost triples by 2030, reaching around 230 kWh per year, and expected to rise to around 560 kWh per year by 2050. By 2050 more than 90% of the population is expected to have access to electricity, either via the main grid (82%) or a mini-grid (10%), the rest using solar home systems.

Currently about 95% of the population of Uganda primarily cook using solid biomass, whereas only the remaining 5% use LPG, electricity (mostly in the urban areas of Kampala and the Central Region), or biogas. Affordability is a fundamental obstacle to the swich to clean cooking, considering that less than 30% of the population without access to such technology, are financially able to afford using LPG, and only 10% an electric hotplate. The ETP reflects this situation in the pace of adoption of clean cooking throughout the planning horizon. It is assumed that about 15% of the population gaining access to electricity by 2030 will use electricity for cooking, and about 50% of the total population by 2050.

Figure 7  Share of households by cooking technology, 2021-2050

Source: Uganda Energy Transition Plan

##### Buildings

Thanks to its climate conditions, both heating and cooling energy needs of buildings in Uganda are relatively low. Energy needs through the planning horizon reflect the rise of population, urbanisation, and living standards, accompanied by increased use of electric appliances.

Figure 8  Building sector final energy consumption by fuel and sub-sector, 2021-2050

Source: Uganda Energy Transition Plan

Energy Efficiency improvements are expected to accompany this trend, as minimum energy performance standards are assumed to be introduced, along with the Energy Efficiency and Conservation Bill, which is approved by the Government but not yet introduced for discussion in the Parliament.

##### Agriculture

By 2050, electricity is set to account for 45% of energy demand in agriculture, thanks to a shift towards more sustainable solutions such as efficient electric pumps for irrigation.

Figure 9  Agriculture sector final energy consumption by fuel, 2021-2050

Source: Uganda Energy Transition Plan

##### Industry

Figure 10  Industry energy demand by fuel and sub-sector, 2021-2050

Source: Uganda Energy Transition Plan

#### Demand for Electricity in the ETP

The ETP includes an assessment of the electricity demand by sector, based on the above assumptions and policy intentions, illustrated in Figure 11. The forecast reflects the increasing electrification and incorporates the relatively high efficiency of new appliances and equipment installed by new customers as the country is electrified.

Figure 11  Electricity demand by sector, 2010-2050

Source: Uganda Energy Transition Plan

#### Requirements for implementation of the ETP

The funds required for implementation of the ETP amount to

investments of USD 850 million per year to 2030, of which nearly 90% targets electricity access and the remainder for clean cooking

annual clean energy investments of around USD 7.2 billion by 2030, spread across electricity access, power generation, grid enhancements and end-use measures like access to clean cooking

annual investment of around USD 1.2 billion for implementation of Energy Efficiency measures and the electrification of end uses, including Energy Efficiency retrofits of public buildings, developing electrified public transport, and industrial Energy Efficiency modernisation.

The following list summarises the requirements for implementation of the targets of the ETP:

regional integration

managing affordability

climate and system resilience

institutional capacity building

skilled workers

Specific actions and measures by sector, relative to EE and DSM include the following (copied from the Roadmap)

##### Access to electricity

Implement a robust system for tracking and disseminating access to electricity indicators, incorporating technology and geolocation details as part of the process. This comprehensive approach ensures better governmental planning and provides a valuable tool for the private sector to prioritise investments. Both demand-side (e.g. household surveys) and supply-side (e.g. energy companies) data sources should be used in a sustainable data strategy.

Ensure frequent updates to national electrification plans, monitoring progress based on the latest data and making details available to relevant stakeholders, including the private sector and investors.

Strengthen, expand, and precisely target end-use affordability measures, extending beyond grid social tariffs to include off-grid solutions to ensure that everyone can afford basic electricity services. Special rates can be offered to households that adopt electric cooking, electric agriculture pumps, and electric vehicles to reinforce efforts for consumers to adopt these technologies.

Provide affordability support or payment plans that help low-income consumers cover upfront costs related to securing a connection or installing an off-grid system.

Develop policies that foster a fair market environment for decentralised systems, including tax reductions and import tariff exemptions for components dependent on adherence to quality standards, grid interconnection and interoperability standards. Provide appropriate protections for mini-grid operators to have reasonable line-of-sight on operating timelines, licenses, and terms if the project is integrated into the grid.

Work with financial institutions to develop standard models for access financing to streamline investments for grid extensions and off-grids systems, including models like mass procurement of solar home systems.

Co-ordinate with clean cooking and other development initiatives to leverage synergies for a more significant impact on energy use.

##### Access to clean cooking

Ensure the National Clean Cooking Strategy, currently under development, includes provisions for all relevant clean cooking technologies tailored to different consumer categories and locations.

Reinforce institutions working on clean cooking and strengthen their mandate to engage across agencies to address gaps in clean cooking.

Develop an implementation plan that incorporates best practice on clean cooking programmes, including peer-to-peer learning, awareness campaigns, mass procurement, and building up local supply chains.

Deploy specific incentives that cover both the adoption of new stoves and the acquisition of fuel, employing diversified approaches based on technologies and well targeted to households and communities most in need. This is particularly important for households currently relying on free collected biomass.

Develop LPG infrastructure such as distribution networks, storage and refilling stations to ensure availability of supply.

Strengthen information campaigns to promote the benefits of more efficient cooking with tailored strategies to local traditions to address barriers rooted in cultural practices. Complementing measures to adopt include extensive training for stove use, safety education, cooking classes, and recipe books to facilitate the transition to new cooking methods.

Develop and enforce quality standards for stoves and fuels, coupled with incentives that direct consumers towards safe, long-lasting and efficient products.

Establish a national legal framework for accessing carbon credits markets specifically for clean cooking projects. These efforts need to be aligned with international standards, including markets developed under Article 6 of the Paris Agreement. This encourages sustainable practices and provides additional financial support for clean cooking initiatives.

##### Buildings

Pass the Energy Efficiency and Conservation Bill to create the first legal framework for Energy Efficiency regulations in buildings and other sectors, and their enforcement.

Approve the National Energy Efficiency and Conservation Strategy and Plan, due to be finalised following the adoption of the draft Energy Efficiency and Conservation Bill.

Strongly enforce Minimum Energy Performance Standards for appliances such as refrigerators, air conditioners and lighting to reduce energy demand, with a particular focus on second-hand imports.

Encourage the purchase of energy-efficient and electric appliances with financial incentives, especially for low-income households.

Create and strictly enforce building energy codes for new construction and major retrofits to ensure energy efficiency. In addition, green building practices should be encouraged through tax rebates and other financial incentives.

Utilise digital technologies for real-time energy monitoring and management in buildings to enable future demand-response capability.

Implement widespread awareness campaigns on Energy Efficiency measures and benefits in residential and commercial buildings.

Introduce mandatory energy audits for buildings to ensure compliance with efficiency regulations.

Facilitate the growth of energy service companies (ESCOs) to finance and implement energy-saving measures in buildings.

##### Agriculture

Encourage the use of energy-efficient electric pumps for irrigation, combined with solar PV solutions, through financial incentives and trainings.

Facilitate the transition from diesel to electric-powered agricultural processing equipment.

Invest in and support the development of efficient cold chain infrastructures for agriculture.

Offer incentives for adopting energy-efficient and sustainable agricultural practices.

Support the formation of co-operatives to provide affordable and efficient energy solutions for small-scale farmers.

Encourage the use of bioenergy in agricultural processes where appropriate, facilitated, for example, through clusters that combine agricultural activities with energy production, like biogas from waste.

##### Transport

Implement the National E-Mobility Strategy, currently under development, including vehicle purchase incentives and regulatory frameworks.

Invest in modern and efficient public transport systems, including buses and rail, to reduce reliance on personal vehicles, thereby decreasing overall transportation energy demand.

Focus on urban planning strategies that favour walkable, bike-friendly cities and reduce the need for long commutes.

Offer subsidies or tax rebates for the purchase of electric vehicles (EV), particularly targeting two- and three-wheeler vehicles, to encourage their adoption over traditional internal combustion engine vehicles.

Prioritise the development of comprehensive electric vehicle charging networks, focusing on urban areas and major transport corridors to support the growing EV market.

Launch a government-led initiative to electrify its own vehicle fleet as a model for wider adoption. In parallel, private operators should be encouraged to transition to electric buses and light commercial vehicles through favourable procurement policies and financial incentives.

Implement progressive taxation policies where higher taxes are levied on fossil fuel-based vehicles while offering tax exemptions or reductions for electric and hybrid vehicles.

Launch campaigns to educate the public on the benefits of EVs and energy efficient transport, with a particular emphasis on public transport.

Set and enforce rigorous fuel economy standards for all new vehicles, gradually phasing out less efficient models.

Implement low-emission days or zones restricting access in urban areas for high-emitting vehicles to encourage the use of cleaner transport options.

Support the development and usage of biofuels in aviation and heavy-duty freight, where electrification remains challenging.

##### Industry

Implement Energy Efficiency assessments for industrial facilities designed to recommend efficiency measures, notably efficient electric motors, electrifying heating, and improving heat recovery systems.

Introduce electric heat pump units for low-heat processes in industrial sectors such as food drying and processing.

Develop a road map for industrial infrastructure development, such as rail extensions, so industrial players can make informed decisions on their long term expansion plans.

Develop a long-term industrial plan to reach carbon neutrality in each major industry, identify near-term emissions reduction opportunities and ensure new long-lifetime assets are compatible with national decarbonisation targets.

Advance dematerialisation codes to minimise the demand for construction materials where possible.

Optimise for lower clinker-ratios in cement use, which would scale depending on the specific cement application to not sacrifice on performance.

Introduce electric kilns or upgrade existing kilns for cement production with more modern types.

Ensure new direct reduced iron (DRI) steel facilities can accommodate alternatives to fossil fuels, such as green hydrogen, in order to further abate fossil fuel emissions from high-temperature heat processes.

Increase the use of electric arc furnaces (EAF) to reduce reliance on coking coal and to increase the flexibility of feedstocks. In parallel, cultivate a robust scrap steel recovery programme that guards against risks for illegal scrap trade.

# EE and DSM prospects in Uganda

Demand-side management includes all measures taken on the demand side of the power system, affecting the “normal” behaviour of demand. This can include simple measures, such as increasing the efficiency of a light bulb by replacing an incandescent light bulb with a fluorescent one, to using sophisticated controllers to dynamically change the load.

Demand-side measures can be broadly categorised into two different categories:

Energy Efficiency (EE) – includes efficiency improvement of the end-use appliances or of the end user’s premises, so that energy losses are reduced. This approach towards achieving demand-side management is related to technical features, rather than changes in the consumer’s personal comfort and behaviour.

Demand Response (DR) – DR gives an opportunity for consumers to take part in the operation of the electric grid by shifting or regulating their electricity consumption (mainly reducing it during peak time) in response to specific (mainly financial) incentives (price based or incentive based).

- DR programs are typically used by electric operators and system planners as one among other cost-effective options to balance supply and demand. These programmes incentivise customers to change their energy consumption patterns.

Today’s electricity systems complement their operation with distributed energy resources, such that these provide flexibility in emergency or other grid congestion situations. In this context, demand side management rises as an attractive solutions for easy flexibility implementation in power systems.

### Demand Response options

DR allows electricity customers to actively participate in the operation of the electric grid by shifting or appropriately modifying their electricity demand, in particular during peak time, in response to specific financial incentives. DR options include retail price based and incentive based programs.

The following table describes several options available in electricity markets for DR measures (both retail price and incentive based) and comments on their applicability in Uganda to conclude on DR measures that could be considered as potential options for the IERMP.

Table 4  DR measures

| DR Measure | Descriptions / Comments |
| --- | --- |
| Retail price based DR |  |
| 1. ToU tariffs | Retail price-based programmes mainly concern implementation of time of use tariffs, varying by hour or season, incentivising customers to use electricity during non-peak hours, or away from periods of energy shortages. Such programmes would require the introduction of meters capable of registering the consumption of electricity at different times. Such programmes ideally relate to the operation of a cost reflective wholesale electricity market, and would maximise positive impacts where available for large industrial customers. Uganda has already implemented Time of Use tariffs for commercial and industrial consumers for three time of use periods (Peak, Shoulder and Off-peak). Additionally, a cost of service study is currently under implementation seeking to improve the efficiency of the tariffs. Residential consumers are the only customers that do not have time of use tariffs and time of use tariffs could be considered as an option to improve demand response. However, considering the cost of needed metering equipment for domestic customers, and the priority for Uganda at this point to extend electrification to the significant part of the population, this option is not considered applicable for the domestic sector at present. |
| Incentive based DR |  |
| 1. Direct load control | The utility or network operator may remotely shut down or cycle a customer’s electrical equipment (eg air conditioner, water heater), on short notice, to address system or local reliability contingencies. Customers often receive a participation payment, usually in the form of an electricity bill credit. A few programs provide customers with the option to override or opt-out of the control action. However, these actions almost always reduce customer incentive payments. Direct load control programs are primarily offered to residential and small commercial customers. Considering the very low average consumption in the domestic and commercial sector of Uganda, as well as the complexity of the operation and the increased capital cost requirements of such systems, they are not considered applicable in the country at this stage. |
| 2. Interruptible / curtailable (I/C) service | This concerns programs integrated with the customer tariff providing a rate discount or bill credit for agreeing to reduce load, typically to a pre-specified firm service level during system contingencies. Customers that do not reduce load typically pay penalties in the form of very high electricity prices that come into effect during contingency events or may be removed from the program. Such arrangements could be examined by UETCL with large industrial users, also considering any current practices on unpaid load shedding. |
| 3. Demand bidding / buyback programs | These are programs targeted mainly to large customers, encouraging them to either bid into a wholesale electricity market and offer to provide load reductions at a price at which they are willing to be curtailed, or identify how much load they would be willing to curtail at a utility-posted price. Such tailor-made programs may be suitable for and of interest to certain large industrial customers, and should be further examined by UETCL. |
| 4. Emergency demand response programs | Programs that provide incentive payments to customers for measured load reductions during reliability-triggered events; emergency demand response programs may or may not levy penalties when enrolled customers do not respond. The type of metering equipment required for this type of interventions might only concern certain large industrial customers in Uganda. Such arrangements could be examined by UETCL with large industrial users, also considering any current practices on unpaid load shedding. |
| 5. Capacity market programs | These programs are typically offered to customers that can commit to providing pre-specified load reductions when system contingencies arise. Customers typically receive day-of notice of events. Given the current market structure in Uganda, this measure is not applicable. |
| 6. Ancillary Services Market Programs | These programs allow customers to bid load curtailments in open markets as operating reserves. If their bids are accepted, they are paid the market price for committing to be on standby. Similarly, given the current market structure in Uganda, this measure is not applicable. |

Source: ECA analysis

### Energy efficiency options

The domestic, commercial and public services sectors appear to have significant potential for Energy Efficiency (EE) improvement, considering the very high proportion of biofuels and waste in their energy consumption (mainly cooking and hot water preparation), and very low access to and use of electricity, especially in rural areas.

#### Domestic sector

Access to electricity and clean cooking are a priority in the country’s energy policies. According to the Energy Policy for Uganda 2023, the objective is to add 300,000 new connections to the grid annually, and achieve a 60% rate for access to electricity by 2027. Also, to increase the proportion of the population using clean cooking fuels and technologies to 50% by 2040.

Regarding the EE improvement potential of households having access to electricity, the following may be considered. New customers, especially in urban centres, consume electricity at a significantly higher level than the country average, probably reflecting a higher economic standard. The size of the energy saving potential of these customers would depend on both, the share of the different end-uses in their total consumption, and the type and condition of electric appliances used by them.

It may be assumed that this type of new customers, ie newly connected customers with an average annual electricity consumption in the order of 2,875 kWh, have already equipped their houses or flats with relatively modern appliances, ie with little room for significant EE improvement. This is especially the case with lighting, where, as confirmed by the Energy Use survey conducted for the purposes of the study in January 2025, and shown in Table 5, 46% of the households connected to the grid are using LED lamps and 28% CFL lamps for lighting, hence there is very little potential for Energy Efficiency improvement through replacement of incandescent lamps, still used by about 21% of households.

Table 5  Energy demand for lighting per household

| Type of bulb | Type of household | Average number of lamps | Average capacity of lamp (kW) | Days of use per year | Hours of use per day | Energy demand for lighting (kWh per year) | Share of households (% of total households) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CFL lightbulb | Urban | 4.8 | 0.011 | 365 | 4 | 77.2 | 22% |
|  | Rural | 3.9 | 0.011 | 365 | 4 | 63.3 | 6% |
| LED lightbulb | Urban | 4.8 | 0.007 | 365 | 4 | 49.2 | 33% |
|  | Rural | 3.9 | 0.007 | 365 | 4 | 40.3 | 13% |
| Incandescent lightbulb | Urban | 4.8 | 0.040 | 365 | 4 | 280.9 | 14% |
|  | Rural | 3.9 | 0.040 | 365 | 4 | 230.1 | 7% |
| Weighted average | Urban | - | - | - | - | 105.1 | - |
|  | Rural | - | - | - | - | 96.7 | - |

Source: Uganda Energy Use Survey data, January 2025

It may also be assumed, that, where such electric appliances are relatively new, users would not be keen to replace them before some time, even until they reach the end of their useful life.

Current low consumption customers (mainly rural) have practically no potential for EE improvement from their electric uses. Rather, they have potential for increased energy consumption as living standards rise, and they may be expected to afford higher expenditure for energy. Higher energy consumption will mean adding further energy uses compared to current basic ones (eg more lighting, clean cooking, air conditioning, etc). As found by the Energy Use survey, there is practically no use of electric appliances for spacing heating and/or cooling (also partly reflecting Uganda’s mild climatic conditions), and only a negligible share of households (significantly lower in rural areas) are using refrigerators, freezers, hair dryers, etc (see Table 6).

Table 6  Energy consuming appliances used by households

| Appliance/ Type of household | Number of appliances | Average capacity (kW) | Number of times used per week | Duration of use per time (hours) | Current usage (% of urban or rural households) |
| --- | --- | --- | --- | --- | --- |
| Electric radiator |  |  |  |  |  |
| Urban | 0 | 2.00 | 0.00 | 0.00 | 0% |
| Rural | 0 | 2.00 | 0.00 | 0.00 | 0% |
| Air conditioning |  |  |  |  |  |
| Urban | 0 | 3.25 | 0.00 | 0.00 | 0% |
| Rural | 0 | 3.25 | 0.00 | 0.00 | 0% |
| Electric fan |  |  |  |  |  |
| Urban | 2 | 0.12 | 2.00 | 2.00 | 8% |
| Rural | 2 | 0.12 | 2.00 | 2.00 | 3% |
| Mobile phone charging |  |  |  |  |  |
| Urban | 2 | 0.00 | 15.20 | 3.22 | 70% |
| Rural | 2 | 0.00 | 12.91 | 3.25 | 59% |
| TV |  |  |  |  |  |
| Urban | 1 | 0.02 | 14.25 | 2.95 | 43% |
| Rural | 1 | 0.02 | 11.31 | 3.10 | 30% |
| Radio |  |  |  |  |  |
| Urban | 1 | 0.01 | 12.39 | 2.42 | 31% |
| Rural | 1 | 0.01 | 13.08 | 2.75 | 28% |
| Iron |  |  |  |  |  |
| Urban | 1 | 1.00 | 3.65 | 2.47 | 35% |
| Rural | 1 | 1.00 | 3.44 | 2.47 | 15% |
| Kettle |  |  |  |  |  |
| Urban | 1 | 1.20 | 10.16 | 1.77 | 13% |
| Rural | 1 | 1.20 | 9.00 | 2.00 | 6% |
| Refrigerator |  |  |  |  |  |
| Urban | 1 | 0.10 | 9.69 | 3.71 | 6% |
| Rural | 1 | 0.10 | 7.20 | 4.17 | 1% |
| Computer |  |  |  |  |  |
| Urban | 1 | 0.05 | 7.00 | 1.50 | 8% |
| Rural | 1 | 0.05 | 7.00 | 1.50 | 1% |
| Chest freezer |  |  |  |  |  |
| Urban | 1 | 0.03 | 9.69 | 6.00 | 6% |
| Rural | 1 | 0.03 | 7.20 | 5.00 | 1% |
| Refrigerator with freezer |  |  |  |  |  |
| Urban | 1 | 0.15 | 9.69 | 6.00 | 6% |
| Rural | 1 | 0.15 | 7.20 | 5.00 | 1% |
| Hair dryer |  |  |  |  |  |
| Urban | 1 | 1.50 | 7.00 | 0.08 | 2% |
| Rural | 1 | 1.50 | 7.00 | 0.08 | 1% |

Source: Uganda Energy Use Survey data, January 2025

It should be expected that the appliances purchased for such new uses, will be in accordance with energy performance and labelling regulations (which are expected to be adopted and applied), ie will have little if any potential for improvement of EE, therefore, cannot be expected to be substituted by higher standard ones before the end of their useful lives.

Customers to be connected in the future, along the high connection rates targeted, should be expected to use energy efficient electric appliances both as a result of applicable energy performance and labelling regulations, and as a matter of economic choice, considering these would be economically superior compared to low standard ones. Therefore, their energy consumption should be expected to reflect average consumption using efficient equipment.

Economic growth and higher income levels are expected to increase the average electricity consumption. According to the 2023 Energy Policy, the target is for the annual electricity consumption per capita to rise to 578 kWh by 2040. At the same time, energy savings potential may be expected to drop, as the appliances installed by newly connected final customers conform to applicable energy performance standards and labelling regulations.

#### Commercial and public buildings

Energy saving opportunities for commercial and public buildings will be mainly relevant to existing connections, as new ones can be expected to use energy efficient appliances, subject to relevant standards. Such EE improvements for existing customers would focus on lighting, where high efficiency or led lighting have not been introduced already, air-conditioning, refrigeration, washing etc. Opportunities to improve the EE will also relate to substitution of low efficiency cooking systems with clean cooking ones.

#### Industry and transport

Energy saving opportunities may also be considered for industry, although it may be expected that competition would drive industries towards utilising available technology, for minimising their energy costs.

The potential for EE improvements in transport would mainly relate to adoption of high efficiency standards for new vehicle, including adoption of electric vehicles, improvement of the average age of vehicles, and improvement and extension of mass transport media.

# EE and DSM candidate options in Uganda

The Energy Efficiency Roadmap for Uganda, prepared in 2017, distilled EE and DSM measures that should be implemented in Uganda. It also assessed the technical, economic and achievable energy savings potential in the country of each measure. For the determination of the achievable energy savings potential the Roadmap used available data and assumptions on the current and prospective electricity consumption by sector and end-use, coincidence factors, energy prices, etc. The Roadmap further recommended to assess the identified measures in an IRP to conclude which of the identified options are least cost options to pursue for the energy sector.

The average electricity savings, cost, and lifetime of electric appliances candidate for replacement of inefficient ones used in the domestic, commercial, and industrial sectors is shown in Table 7 - Table 9.

Table 7  Electricity savings, cost, and lifetime of candidate energy efficient appliances to be used in the residential sector (ranked by incremental cost of savings)

| Measure | Average electricity savings (%) | Measure lifetime (year) | Incremental cost per average annual savings (USD/kWh) |
| --- | --- | --- | --- |
| LED lighting | 58 | 10 | 0.10 |
| Heat-pump hot water heater | 63 | 8 | 0.19 |
| High-efficiency fan | 55 | 2 | 0.25 |
| High-efficiency refrigerator | 50 | 6 | 0.52 |
| Solar hot water heater | 75 | 8 | 0.57 |
| High-efficiency room AC unit | 22 | 6 | 0.98 |
| LCD TV | 50 | 2 | 1.58 |

Source: Energy Efficiency Roadmap for Uganda

Table 8  Electricity savings, cost, and lifetime of candidate energy efficient appliances to be used in the commercial sector (ranked by incremental cost of savings)

| Measure | Average electricity savings (%) | Measure lifetime (year) | Incremental cost [O&M cost] per average annual savings (USD/kWh) |
| --- | --- | --- | --- |
| Low-flow faucet aerator | 55 | 5 | 0.04 |
| Building energy management system | 10 | 8 | 0.07 [0.0045] |
| High-efficiency reach-in cooler | 14 | 5 | 0.09 |
| LED drop-in lighting | 66 | 8 | 0.13 |
| Computer hibernation and auto turn-off | 17 | 1 | 0.15 |
| Heat pump water heater | 63 | 8 | 0.15 |
| Efficient packaged air conditioner | 28 | 8 | 0.16 |
| Variable-speed-drive on chiller system | 19 | 8 | 0.24 |
| High-efficiency solid-door freezer | 56 | 5 | 0.24 |
| Solar water heater | 75 | 8 | 0.28 |
| LED fixture lighting | 66 | 10 | 0.33 |
| Variable-speed-drive on fans | 19 | 5 | 0.34 |
| High-efficiency laptops and desktops | 43 | 2 | 0.54 |
| Network sharing for printers, photocopiers, scanners | 50 | 5 | 0.56 |
| Low solar heat gain window film | 21 | 5 | 0.61 |
| Weatherization: apply blinds, shutters, air-sealing, etc. | 7 | 5 | 1.20 |
| AC energy-saver system | 9 | 4 | 1.95 |

Source: Energy Efficiency Roadmap for Uganda

Table 9  Electricity savings, cost, and lifetime of candidate energy efficient appliances to be used in the industrial sector (ranked by incremental cost of savings)

| Measure | Average electricity savings (%) | Measure lifetime (year) | Incremental cost [O&M cost] per average annual savings (USD/kWh) |
| --- | --- | --- | --- |
| Reduce compressed air tank pressure | 7 | 1 | 0.01 [0.0046] |
| Properly sized motors and drives | 3 | 8 | 0.02 |
| Optimized chilled water temperature | 3 | 1 | 0.02 |
| Reduce compressed air leaks | 20 | 2 | 0.02 [0.0068] |
| Preventative maintenance for motors /drives | 4 | 2 | 0.03 |
| Preventative refrigeration/ cooling system maintenance | 5 | 2 | 0.03 |
| Preventative furnace/boiler maintenance | 5 | 2 | 0.03 |
| Advanced heating and process control | 5 | 3 | 0.05 |
| Energy Management System (EMaS), ISO50001 | 10 | 3 | 0.07 [0.0045] |
| Energy Monitoring System (EMoS), ISO50006 | 5 | 3 | 0.08 [0.0045] |
| Demand- controlled compressed air | 15 | 3 | 0.09 |
| Using variable speed drives on air compressors | 25 | 8 | 0.11 [0.0091] |
| High-efficiency large motors | 6 | 8 | 0.21 |
| LED lighting drop-in | 50 | 8 | 0.21 |
| Variable-speed-drive motors | 25 | 8 | 0.27 |
| Refrigeration plant variable- speed drives | 20 | 8 | 0.27 |
| Improved process heating insulation | 5 | 8 | 0.29 |
| Reduction in steam pressure | 5 | 2 | 0.29 |
| LED lighting fixtures | 50 | 10 | 0.30 |
| High-efficiency small motors | 18 | 8 | 0.43 |
| High-efficiency chiller | 19 | 10 | 0.48 |
| Process cooling pipe insulation | 5 | 5 | 0.50 |
| Lighting control system | 20 | 5 | 0.54 [0.0136] |
| Process heat recovery | 6 | 10 | 0.88 |

Source: Energy Efficiency Roadmap for Uganda

Table 10 presents the technical and economic energy savings potential in 2030, ranked by end-user benefit/cost ratio, in the residential, commercial, and industrial sectors.

Measures that are considered clearly uneconomic (cost/benefit ratio below 1) and measures that are considered not implementable are highlighted with orange background and will be excluded as candidate options for the IERMP analysis. Measures that will be considered in the least cost planning analysis are highlighted with green background.

Measures with a user benefit/cost ratio above 1 are economically feasible from the point of view of the user and will be taken for consideration in the next phase of the IRP as candidate options. DSM and EE measures relative to water heating are not further considered as options for the IRP analysis, as findings from the Energy Use survey indicate that the corresponding end use is currently non-existent, or insignificant.

Industrial users operate under a competitive environment which incentivises them to use efficient devices to remain competitive in the market. Efficiency improvements of industrial consumers have already been incorporated in the demand forecast assuming the historical trend of efficiency improvement. Other measures which are site, process, and volume specific, for industrial users, and energy management systems for the commercial users cannot be implemented as a generalised measure and depend on the discretion of the user to select them depending on market competition. What can be done is to provide incentives to such consumers to use efficient devices through minimum standard regulations which are already developed for Uganda and will be implemented soon.

Table 10  Technical and economic potential savings by measure in 2030, ranked by end-user benefit-to-cost ratio, in each sector

| Measure | Consumption Savings (GWh) | Demand Savings (MW) | Cost of conserved energy (USc/kWh) | End-user benefit cost ratio |
| --- | --- | --- | --- | --- |
| Residential sector |  |  |  |  |
| Urban LED lighting | 197 | 52 | 1 | 12.3 |
| Rural LED lighting | 70 | 23 | 1 | 12.3 |
| Urban heat pump hot water heater | 18 | 2 | 3 | 5.6 |
| Rural heat pump hot water heater | 2 | 0 | 3 | 5.6 |
| Urban solar hot water heater | 63 | 6 | 9 | 1.9 |
| Rural solar hot water heater | 9 | 1 | 9 | 1.9 |
| Urban efficient refrigerator | 163 | 14 | 11 | 1.7 |
| Rural efficient refrigerator | 37 | 4 | 11 | 1.7 |
| Urban efficient fan | 33 | 7 | 14 | 1.3 |
| Rural efficient fan | 10 | 2 | 14 | 1.3 |
| Urban high-efficiency AC | 23 | 5 | 21 | 0.9 |
| Rural high-efficiency AC | 3 | 1 | 21 | 0.9 |
| Urban efficient TV | 70 | 19 | 87 | 0.2 |
| Rural efficient TV | 21 | 7 | 87 | 0.2 |
| Commercial sector |  |  |  |  |
| Low-flow faucet aerator | 18 | 4 | 1 | 13.3 |
| Energy management system | 30 | 5 | 2 | 8.8 |
| High-efficiency reach-in cooler | 5 | 1 | 2 | 6.5 |
| LED drop-in lighting | 95 | 15 | 2 | 6.3 |
| Heat pump water heater | 21 | 5 | 2 | 5.6 |
| Efficient packaged air conditioner | 34 | 3 | 3 | 5.3 |
| Variable speed drive on chiller system | 6 | 0 | 4 | 3.5 |
| Solar water heater | 75 | 18 | 5 | 3 |
| LED fixture lighting | 95 | 15 | 5 | 2.9 |
| High-efficiency solid-door freezer | 18 | 4 | 6 | 2.3 |
| Variable speed drive on fans | 15 | 1 | 8 | 1.7 |
| Network sharing for printers, photocopiers, scanners | 35 | 4 | 14 | 1 |
| Low-solar-heat-gain window film | 12 | 1 | 15 | 0.9 |
| Computer hibernation and auto turn-off | 6 | 1 | 16 | 0.9 |
| Weatherization: apply blinds, shutters, air-sealing, etc. | 4 | 0 | 29 | 0.5 |
| High-efficiency laptops and desktops | 15 | 2 | 30 | 0.5 |
| Air conditioning energy-saver system | 8 | 1 | 58 | 0.2 |
| Industrial sector |  |  |  |  |
| Properly sized motors and drives | 22 | 3 | 1 | 32.8 |
| Reduce compressed air tank pressure | 7 | 1 | 1 | 11.7 |
| Reduce compressed air leaks | 19 | 2 | 2 | 6.8 |
| Preventative maintenance for motors/drives | 28 | 4 | 2 | 6.6 |
| Preventative refrigeration/cooling system maintenance | 6 | 1 | 2 | 6.6 |
| Preventative furnace/boiler maintenance | 15 | 2 | 2 | 6.6 |
| Advanced heating and process control | 16 | 2 | 2 | 5.9 |
| Optimized chilled water temperature | 3 | 0 | 3 | 4.1 |
| Use variable speed drive on air compressors | 32 | 4 | 3 | 4 |
| Energy management system, ISO50001 | 190 | 24 | 3 | 3.6 |
| Demand-controlled compressed air | 14 | 2 | 3 | 3.3 |
| High-efficiency large motors | 42 | 5 | 3 | 3.2 |
| LED drop-in lighting | 27 | 3 | 4 | 3.1 |
| Energy monitoring system, ISO50006 | 95 | 12 | 4 | 3.1 |
| LED lighting fixtures | 27 | 3 | 4 | 2.6 |
| Variable speed drive motors | 276 | 35 | 4 | 2.5 |
| Refrigeration plant variable speed drives | 38 | 5 | 5 | 2.4 |
| Improved process heating insulation | 17 | 2 | 5 | 2.3 |
| High-efficiency chiller | 29 | 4 | 7 | 1.6 |
| High-efficiency small motors | 22 | 3 | 7 | 1.5 |
| Process cooling pipe insulation | 6 | 1 | 12 | 0.9 |
| Process heat recovery | 4 | 1 | 13 | 0.9 |
| Lighting control system | 9 | 1 | 15 | 0.8 |
| Reduction in steam pressure | 4 | 0 | 16 | 0.7 |

Source: Energy Efficiency Roadmap for Uganda

Figure 12 illustrates the measures contributing to the achievable economic energy savings potential, ranked by the end-user cost of conserved energy.

Figure 12  Top Measures for Achievable Economic Potential, Medium Case Prioritized by End-User Cost of Conserved Energy

Source: Energy Efficiency Roadmap for Uganda

#### Further activities to complement the effectiveness of energy efficiency and demand side management measures

It is assumed that for the implementation of energy efficiency interventions, as discussed above, a comprehensive set of underlying policies and other activities is in place, by competent authorities and operators, including -but not limited to- the following:

Complement/update the regulatory framework in the energy sector, to introduce minimum energy performance requirements, energy labelling for energy consuming appliances, and energy efficient building regulations and codes, including energy performance certificates for buildings, etc. Relevant regulation in addition to setting the requirements, conditions, and procedures, should also include monitoring arrangements and appropriate penalties for non-compliance.

Specify the necessary policies and other relevant state interventions to support implementation of the set of supply and demand side measures needed to meet future demand. Setup of an energy efficiency implementation plan would be an effective instrument for implementation of measures identified in the IRP analysis, and this should also include a roadmap for the necessary policies, incentives, etc.

Improve and streamline data collection to serve the purposes of effective interventions in the energy sector, especially regarding end use appliances and patterns in all sectors, to enable targeted interventions by state authorities and other stakeholders, monitor the progress of implementation of policies and interventions relative to improvement of energy efficiency, and update the IRP and related policy instruments. Apart from data collection through dedicated surveys and censuses, other instruments should be also utilised, eg smart meters, energy efficiency certificates issued for buildings, other bilateral partnerships between utilities and customers, as may be introduced and applied.

Raise awareness, including activities aimed at supporting behavioural change relative to energy efficiency through the educational system.

Establish regional collaboration – Although each country is a special case with regard to the specification, implementation and effectiveness of demand-side management, there are often useful lessons to learn from international experience. It would be useful for Uganda to consider participation in relevant regional initiatives, especially with neighbouring countries in the Eastern Africa Power Pool (EAPP). The programmes implemented in the context of the EAPP mostly focus on the development of the regional electricity market, however, through such cooperation it would be possible to establish cooperation schemes on energy efficiency issues with other countries.

Work force development and capacity building – Improvement of energy efficiency requires specialist skills for installation of equipment and mostly for building renovation. Where this is planned to take place on a large scale, specialist training (and possibly licensing) will also need to be planned for the work force that should undertake the relevant activities. Adjustments to curricula of universities and technical institutions would also need to be introduced so that engineers, architects, and technical specialists are equipped with the needed knowledge and skills, as professionals and/or trainers.

Adress energy poverty – Raising energy efficiency in the domestic sector lowers energy costs while maintaining and/or raising living standards, alleviating energy poverty. On the other hand, energy poor households are by definition unable to meet the cost of investment needed to improve energy efficiency at their homes. Policies to assist such households to improve energy efficiency should be considered, eg by providing subsidies for the purchase of energy efficient appliances.

#### Financing issues

The Energy Policy for Uganda 2023 includes considerations relative to energy project financing in Uganda, concluding that a mix of Central and local Government financing, direct private investment, and other sources through bilateral or multilateral programmes, would be necessary for implementation of the Energy Policy objectives.

Apart from direct financing through the national budget, the Energy Policy alco refers to the Medium-Term Expenditure Framework (MTEF), as well as funding from IFIs and the private sector.

The key role of domestic and internationally contributions by private sources is underlined, considering that certain energy projects and programmes have already been implemented through the PPP framework, providing relevant experience and know-how. Moreover, the Government has established syndicates of commercial banks and large surplus institutions to finance PPPs in the energy sector. Such potential financing sources are summarised in Table 11.

Table 11  Financing sources for strategic partnerships

| Service Sectors | Sources of funding |  |
| --- | --- | --- |
|  | Public | Private |
| Public Services Government institutions | Government of Uganda (central and local Government through taxation) Donors/Development Partners - Central Budget Support - District Budget Support - Multilateral and bilateral projects and programmes channelled through central or local Government | Individual entrepreneurs NGO-supported projects and programmes |
| Private Services Domestic private sector International private sector | Government subsidies or cost support to the private sector Contractual arrangements/PPPs with the private sector Participation in Government-funded programmes Multilateral and bilateral projects and programmes channelled through central or local Government | Household (user fees) Donations (internal and external) Income generating activities Fundraising NGO-supported projects and programmes |

Source: Energy Policy for Uganda 2023

# Candidate EE and DSM measures to be analysed in the ERMP

Candidate measures that will be assessed in the IERMP to conclude on the least cost options for implements are shown in the table below. Measures considered clearly uneconomic (cost/benefit ratio below 1) and measures considered not implementable were excluded as candidate options for the IERMP analysis as discussed in the previous subsections.

Table 12  Candidate EE and DSM measures for the IERMP

| Name of measure | Earliest year available | Lifetime | Capital costs per user (US$) | Fixed O&M cost (US$/year) | Maximum penetration rate (% of sector users connected) | Peak demand reduction (% of total annual system peak demand) | Energy demand reduction (% of total annual sector energy demand) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Residential sector |  |  |  |  |  |  |  |
| Urban LED lighting (4.8/user) | 2026 | 10 | 2.98 | 3% of capex | 67% | 16% | 15% |
| Rural LED lighting (2.98/user) | 2026 | 10 | 2.42 | 3% of capex | 87% | 22% | 20% |
| Urban efficient refrigerator (1/user) | 2026 | 6 | 337.5 | 3% of capex | 6% | 2% | 12% |
| Rural efficient refrigerator (1/user) | 2026 | 6 | 337.5 | 3% of capex | 6% | 1% | 11% |
| Urban efficient fan (2/user) | 2026 | 2 | 999.0 | 3% of capex | 8% | 1% | 2% |
| Rural efficient fan (2/user) | 2026 | 2 | 999.0 | 3% of capex | 3% | 2% | 3% |
| Commercial sector |  |  |  |  |  |  |  |
| Low-flow faucet aerator | 2026 | 5 | 6.76 | 3% of capex | 23% | 0.1% | 7% |
| High-efficiency reach-in cooler | 2026 | 5 | 402.30 | 3% of capex | 23% | 0.4% | 1% |
| LED lighting | 2026 | 8 | 29.70 | 3% of capex | 23% | 1% | 20% |
| Heat pump water heater | 2026 | 8 | 3440.15 | 3% of capex | 100% | 1% | 8% |
| Variable speed drive on chiller system | 2026 | 8 | 294.08 | 3% of capex | 23% | 0.4% | 1% |
| Solar water heater | 2026 | 8 | 972.00 | 3% of capex | 100% | 1% | 10% |
| High-efficiency solid-door freezer | 2026 | 5 | 699.30 | 3% of capex | 23% | 0.4% | 3% |

Source: ECA analysis. Note: The cost for each candidate measure has been estimated using available market data. The sources are shown in Annex A2.

As discussed in the previous sections, the industrial sector measures examined in the Roadmap are not included in the list of candidate options for the IERMP. Their implementation depends on the user and market conditions. Thus, it depends on the own initiative of the user to implement them to become economically competitive in the market. It is pointed out, though, that, as shown in Table 10, the energy savings and peak demand impact of such measures may be significant (ie up to 762 GWh and 97 MW, respectively).

# Annexes

Electricity access targets

Table 13 shows the electrification targets defined for each scenario for 2030, 2040, and 2050.

Table 13  Policy targets for electricity access

| Policy document | National electricity access target | Grid electricity access target | Off-grid based access target | Rural access target |
| --- | --- | --- | --- | --- |
| Vision 2040 | 80% by 2040 | - | - | - |
| 2023 Energy Policy Review | 100% by 2040 | 65% by 2040 | - | - |
| Rural Electrification Strategy and Plan | - | - | - | 51% by 2030 100% in 2040 |
| Electricity Connections Policy | Aims at 300,000 new connections annually to reach 3 million new connections by 2027. |  |  |  |
| National Electrification Strategy | 80% by 2040 | 60% by 2027 | 33% by 2027 | - |

Source: Uganda 2023 Energy Policy Review

Capital costs for the implementation of measures

Table 14  Capital costs for the implementation of EE and DSM measure

|  | Price (Ush) | Price (USD) | Capacity (kW) | No per user | Total cost per user (USD) | Source |
| --- | --- | --- | --- | --- | --- | --- |
| Residential |  |  |  |  |  |  |
| On-grid urban LED lighting | 2,300 | 0.62 | 0.007 | 4.8 | 3 | https://tronic.ug/products/7w-tronic-led-bulb-b22-le-0722-dl |
| On-grid rural LED lighting | 2,300 | 0.62 | 0.007 | 3.9 | 2 | https://tronic.ug/products/7w-tronic-led-bulb-b22-le-0722-dl |
| On-grid urban heat pump hot water heater | 12,741,282 | 3,440.15 | 2.500 | 1.0 | 3,440 | https://paylaterug.com/shop/davis-and-shirtliff/solar-solutions/solar-water-heaters/dayliff-hpw150-2-5kw-1ph-all-in-one-heat-pump/ |
| On-grid rural heat pump hot water heater | 12,741,282 | 3,440.15 | 2.500 | 1.0 | 3,440 | https://paylaterug.com/shop/davis-and-shirtliff/solar-solutions/solar-water-heaters/dayliff-hpw150-2-5kw-1ph-all-in-one-heat-pump/ |
| On-grid urban solar hot water heater | 1,500,000 | 405.00 | - | 1.0 | 405 | https://www.easypowerug.com/solar-water-heater-prices-in-uganda/ |
| On- grid rural solar hot water heater | 1,500,000 | 405.00 | - | 1.0 | 405 | https://www.easypowerug.com/solar-water-heater-prices-in-uganda/ |
| On-grid urban efficient refrigerator | 1,250,000 | 337.50 | - | 1.0 | 338 | https://www.jumia.ug/lg-187l-top-freezer-refrigerator-smart-inverter-compressor-multi-air-flow-gn-b202sqbb-214239141.html |
| On-grid rural efficient refrigerator | 1,250,000 | 337.50 | - | 1.0 | 338 | https://www.jumia.ug/lg-187l-top-freezer-refrigerator-smart-inverter-compressor-multi-air-flow-gn-b202sqbb-214239141.html |
| On-grid urban efficient fan | 1,850,000 | 499.50 | 0.026 | 2.0 | 999 | https://jiji.ug/central-division/home-appliances/lg-ceiling-air-purifier-fan-lcf12p-sleep-mode-thinq-dbKZQFUItkKLBR4S5z9POD2A.html?page=1&pos=2&cur_pos=2&ads_per_page=20&ads_count=287&lid=McLk17Ae_VUlkZqt&indexPosition=1 |
| On-grid rural efficient fan | 1,850,000 | 499.50 | 0.026 | 2.0 | 999 | https://jiji.ug/central-division/home-appliances/lg-ceiling-air-purifier-fan-lcf12p-sleep-mode-thinq-dbKZQFUItkKLBR4S5z9POD2A.html?page=1&pos=2&cur_pos=2&ads_per_page=20&ads_count=287&lid=McLk17Ae_VUlkZqt&indexPosition=1 |
| Commercial |  |  |  |  |  |  |
| Low-flow faucet aerator | 25,024 | 6.76 | - | 1.0 | 7 | https://www.jumia.ug/generic-202224mm-kitchen-sink-faucet-tap-nozzle-thread-swivel-aerator-filter-sprayer-kitchen-water-saving-faucet-accessories-231723634.html |
| High-efficiency reach-in cooler | 1,490,000 | 402.30 | - | 1.0 | 402 | https://www.abanista.com/product/spj-300l-beverage-upright-cooler/ |
| LED drop-in lighting | 110,000 | 29.70 | 50.000 | 1.0 | 30 | https://tronic.ug/products/50w-tronic-600x600-led-daylight-panel-fi-elpl-50-dl |
| Heat pump water heater | 12,741,282 | 3,440.15 | 2.500 | 1.0 | 3,440 | https://paylaterug.com/shop/davis-and-shirtliff/solar-solutions/solar-water-heaters/dayliff-hpw150-2-5kw-1ph-all-in-one-heat-pump/ |
| Variable speed drive on chiller system | 1,089,200 | 294.08 | 4.000 | 1.0 | 294 | https://www.jumia.ug/generic-4kw-380v-3-phase-output-variable-frequency-drive-inverter-vfd-motor-converter-243474636.html |
| Solar water heater | 3,600,000 | 972.00 | - | 1.0 | 972 | https://www.easypowerug.com/solar-water-heater-prices-in-uganda/ |
| LED fixture lighting | 110,000 | 29.70 | 50.000 | 1.0 | 30 | https://tronic.ug/products/50w-tronic-600x600-led-daylight-panel-fi-elpl-50-dl |
| High-efficiency solid-door freezer | 2,590,000 | 699.30 | - | 1.0 | 699 | https://kweli.shop/product/hisense-310-liter-upright-freezer/ |

Source: Indicated in the table

National Population and Housing Census 2024, Uganda Bureau of Statistics

Source: Energy Efficiency Roadmap for Uganda, 2017

Source: Energy Efficiency Roadmap for Uganda, 2017

Source: Energy Efficiency Roadmap for Uganda, 2017